MPPE/MEA MDP Software Spec

Bepi-Colombo MEA & MDP Team

Ver 7.0

2017.12.20

*Shared data

short Gs_TRGBUF_EFD_Pot[ ]; from EFD (1-spin average)

short Gs_TRGBUF_MGFO_Bx[ ] from MGFO – Bx (1-spin average)

short Gs_TRGBUF_MGFO_By[ ] from MGFO – By (1-spin average)

short Gs_TRGBUF_MGFO_Bz[ ] from MGFO – Bz (1-spin average)

unsigned short Gus_TRGBUF_MEA_T[ ]; from MEA

０．Open

１．Development system

MDP:		Kasaba（Tohoku Univ.）

MPPE:		Yokota（ISAS）

２．Command: MEA task in MDP SW

CMD：	Ground　>　DMC　>　MDP(MEA1&2)

| CMD (MD1U_**) MEA1_: 0x05; MEA2_0x06 |  | HK# | Description |
| --- | --- | --- | --- |
| MEA1/2_INIT | 0x0030 |  | Initialize SW parameters |
| MEA1/2_ERR_CLR | 0x0031 |  | Clear the counters |
| MEA1/2_ROM_READ | 0x0032 | 17 | Obtain the parameters from E2PROM |
| MEA1/2_PROC_SNAPSHOT | 0x0010 | 3 | Select SNAPSHOT for multi-spin products |
| MEA1/2_PROC_ AVERAGE | 0x0011 | 3 | Select AVERAGE for multi-spin products |
| MEA1/2_DATA_MODE zz xx | 0x0012 | 3 | Set MPPE mode zz = 0/1  SCI/ENG; xx = 0-8 for SCI and 0-1 for ENG |
| MEA1/2_POT_MODE 00 xx | 0x0013 | 4 | Set the mode for the satellite potential xx = 00/01  0/Sp1 (Sp1 = A*p + B ) |
| MEA1/2_POT_PARAM zz xx aa bb | 0x0015 | 5 | Set A of Sp1 for the satellite potential A= (z << 8) +xx; B = (a << 8) +bb (12bits) #1 |
| MEA1/2_SET_ETPAP zz xx aa bb cc dd ee ff gg | 0x0016 | 12 | Set en# of EtPAP[4en][16ch] and EtPAPe[8en][16ch] 4 or 8 start points of en# and width: zz: width (zz = 0  1 en) en = xx aa bb cc (& 0x1f) used for EtPAP; en = xx aa bb cc dd ee ff gg (& 0x3f) used for EtPAPe; en# = xx - xx + zz, aa - aa + zz, … |
| MEA1/2_SET_VM_AL zz xx aa bb | 0x0018 | 14 | Set the flag of using energy step # for VM[0](…) zz: 31-24; xx: 23-16; aa: 15-8; bb: 7-0# of 32 energy steps 0/1  NOT USE/USE Ex. “ff ff ff fe” does not use the energy step# of 0. |
| MEA1/2_SET_ETBL 00 xx | 0x0019 | 6 | Set the table# of energy(velocity) sweep for VM cal. xx = 1-3 T1-T3 (if xx = 0, xx goes to 1.) Should follow the MEA CMD of changing energy sweep table. |
| MEA1/2_SET_HK_MON 00 xx | 0x0020 | 6 | Set MEA1/2 HK monitoring 1/0 ENA/DIS xx = [][][][4][3][2][1][0] (ex. 00 00  all DIS, 00 1f  all ENA) [0]: MEA1/2_HK_TMP_MCP [1]: MEA1/2_HK_MCP [2]: MEA1/2_I_MCP [3]: MEA1/2_HK_TMP_FPG [4]: MEA1/2_HK_I_P5V |
| MEA1/2_SET_TMCP_LIM zz xx aa bb | 0x0021 | 7 | Set the lower/upper limits of MEA1/2_HK_TMP_MCP (z << 8) +xx : lower limit; (a << 8) + bb : upper limit (12bits) |
| MEA1/2_SET_MCP_LIM zz xx aa bb | 0x0022 | 8 | Set the lower/upper limits of MEA1/2_HK_MCP (z << 8) +xx : lower limit; (a << 8) + bb : upper limit (12bits) |
| MEA1/2_SET_IMCP_LIM zz xx aa bb | 0x0023 | 9 | Set the lower/upper limits of MEA1/2_I_MCP (z << 8) +xx : lower limit; (a << 8) + bb : upper limit (12bits) |
| MEA1/2_SET_TFPG_LIM zz xx aa bb | 0x0024 | 10 | Set the lower/upper limits of MEA1/2_HK_TMP_FPG (z << 8) +xx : lower limit; (a << 8) + bb : upper limit (12bits) |
| MEA1/2_SET_IP5V_LIM zz xx aa bb | 0x0025 | 11 | Set the lower/upper limits of MEA1/2_HK_I_P5V (z << 8) +xx : lower limit; (a << 8) + bb : upper limit (12bits) |
| MEA1/2_SET_STPE_AUT 00 xx | 0x0026 | 3 | Set automatic sending of ‘MEA1/2_StpEnMd_ON’ENA/DIS xx = 1/0 ENA/DIS In case, MDP sends MEA1/2_ StpEnMd_ON (0x11) 0x01-0x20 |
| MEA1/2_SND_GF_AUT zz xx | 0x0027 | 3 15 | Set automatic sending of ‘MEA1/2_G_Factor’ ENA/DIS zz: X (number of spin); xx = 1/0 ENA/DIS In case, MDP sends MEA1/2_G_Factor(0x08) 0x00-0x04 |
| MEA1/2_SET_MCPV zz xx | 0x0028 | 17 | Set V1 and V2 to use before MDP turn off the MCP zz: V1; xx: V2 |
| MEA1/2_ETP_MODE zz xx | 0x0029 | 18 | Set the mode for sector# of EtPAP zz = 1/0  use sector # from B(MGF) + offset(xx)/xx(0-7) |
| MEA1/2_CNT1_SET zz xx aa bb … (for debug) | 0x0002 | 18 | 00 01 aa bb cc dd ee: #SW cal aa(0/1) = DIS/ENA; bb = en; cc = ch; dd = sc; ee = count 00 04 aa bb: #prohibit product compression if (bb) 00 02 aa bb: #send CMD(aa bb) to MEA 00 07 00 bb #limit the size of H-mode product if (bb): |

MEA1/2 changes the parameters of MEA1 or MEA2.

#1) Actual value(V) = (VAL – 0x07ff) x 100/0x07ff (VAL = A, or B)

CMD：	MDP　>　MEA

| CMD | Description |
| --- | --- |
| MEA1/2_StpEnMd_ON 0x11 aa(energy) aa = 0-63 | #Send the energy step number at each spin The energy number is determined by the Satellite Pot x 2 – 5 steps (to protect the MCP from photoelectrons) |
| MEA1/2_G_Factor 0x08 aa(GF_value) aa = 0x00-0x04 | #Decrease g-factor when: Count > 10^5/s/ch continuously during X spins # Increase g-factor when: Count/spin/16chs < 1000 continuously during X spins (X is changeable by CMD.) MEA1: [default  20  60  250  1000] MEA2: [default  1  3  20  60] *GF1&T3 *When MEA1 backs to 60, MEA2 also goes to 60. |

Emergency

３．HK

・HK		20 Byte

-MDP monitors the following parameters at 1Hz.

-MDP sends CMDs when HK_mon = ENA && (HK < lower limit || HK > upper limit).

MEA1/2_HK_TMP_MCP (Upper/lower limits are changeable by CMD)

MEA1/2_HK_MCP (Upper/lower limits are changeable by CMD)

MEA1/2_I_MCP (Upper/lower limits are changeable by CMD)

#MEA1/2_MCP_ON(0x1c) V1

#wait for 4s

#MEA1/2_MCP_ON(0x1c) V2

#wait for 4s

#MEA1/2_MCP_ON(0x1c) 0

#wait for 4s

#MEA1/2_MCP_OFF (0x1900)

MEA1/2_HK_TMP_FPG (Upper/lower limits are changeable by CMD)

MEA1/2_HK_I_P5V (Upper/lower limits are changeable by CMD)

# MEA1/2_MCP_ON(0x1c) V1  V2  0  MEA1/2_MCP_OFF(0x1900) as the above

#wait for 12 s

#MEA1/2 PSU OFF (DmcRequestCmd)

・User-HK	128 Bytes	(No parameter is monitored by MDP)

・Software HK	52Bytes        MEA1/2: Individual parameters

| Name | Description | Byte |
| --- | --- | --- |
| 0. MEA1/2_S_CMD_CTR | Software Command counter | 1 |
| 1. MEA1/2_S_ERR_CTR | Software error counter | 1 |
| 2. MEA1/2_S_CMD_ANS | Software Command answer | 4 |
| 3. MEA1/2_S_DATA_MODE | [7]: 0/1 = DIS/ENA SND_GF_AUT [6]: 0/1 = DIS/ENA SND_STPE_AUT [2-5]: Mode [1]: 0/1 = SCI/EN [0]: 0/1 = average/snapshot | 1 |
| 5. MEA1&2_S_POT_PARAM | 1.5B: A; 1.5B: B | 3 |
| 6. MEA1/2_S_HK_MON (& Energy table number) | [7] POT_MODE [5-6]: Energy table number [4]: MEA1/2_HK_I_P5V [3]: MEA1/2_HK_TMP_FPG [2]: MEA1/2_I_MCP [1]: MEA1/2_HK_MCP [0]: MEA1/2_HK_TMP_MCP 0/1 = DIS/ENA | 1 |
| 7. MEA1/2_S_TMCP_LIM | Lower limit and upper limit (1.5B+1.5B) | 3 |
| 8. MEA1/2_S_MCP_LIM | Lower limit and upper limit | 3 |
| 9. MEA1/2_S_IMCP_LIM | Lower limit and upper limit | 3 |
| 10. MEA1/2_S_TFPG_LIM | Lower limit and upper limit | 3 |
| 11. MEA1/2_S_IP5V_LIM | Lower limit and upper limit | 3 |
| 12. MEA1/2_S_ETPAP | zz xx aa bb cc dd ee ff gg zz: width; others: starting points ex. for (i = xx; i < xx + zz; ++i) | 8->9 |
| 13. MEA1/2_S_VM | zz xx aa bb cc dd | 6->0 |
| 14. MEA1/2_S_VM_AL | zz xx aa bb | 4 |
| 15. MEA1/2_S_GF_AVAL | X (spin number) for g-factor control | 1 |
| 16. MEA_S_HK_ANMLY (MDP did [5] or detected [4-1] is out of the range) | [5]: PSU-OFF [4]: MEA1/2_HK_I_P5V [3]: MEA1/2_HK_TMP_FPG [2]: MEA1/2_I_MCP [1]: MEA1/2_HK_MCP [0]: MEA1/2_HK_TMP_MCP | 1 |
| 17 MEA1/2_S_MCPV | V1 and V2 to set before MDP turn off the MCP | 2 |
| 18. MEA1/2_S_PARAM | 0-[7] spare 0-[6]: B(MGF) Mode (1/0: Use MGF_B/CMD_B) 0-[5-3]: B(sector) for EtPAP 0-[2]: prohibit product compression 0-[1]: SWcal ENA(1)/DIS(0) 0-[0]: Read the parameters of E2PROM (TRUE/FALSE) 0/1 = DIS/ENA 1: 0xca 2: 0xe1(MEA1), 0xe2(MEA2) | 3 |
|  | TOTAL | 46 |

MEA1/2: Individual paremeters

4．Mission data

MDP prepares the mission packets according to the MPPE mode in the following formats (Table MDPSW-MEA-3) in modes for the nominal science observation and an engineering application. MEA1&2 have several modes but use only two modes (M2, 4) in space and thus provides two sorts of 16-bit count data each spin (4s) to MDP (Table MDPSW-MEA-4). In the nominal observation (M2), MEA1&2 have one sort of mission packet for L and H-mode products, while two sorts of M-mode packets are prepared for MEA1 according to the MPPE mode. MDP calculates VM(velocity moments) including density(n), velocity(V), pressure tensor(P) and heat flux vector by using lookup table. In default, MDP has simple lookup table. When sending “MEA1/2_ROM_READ”, MDP reads the parameters stored in EEPROM (Table MDPSW-MEA-5), and produces calibrated lookup table. Configuration of the lookup table is shown in Table MDPSW-MEA-6. All the components of the MEA mission packets are described in Table MDPSW-MEA-7.

Table MDPSW-MEA-3 MEA mission packet (Science obs.)

|  |  | MEA operation mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | M2 (32en x 16sc) |  | M4 (64en x 16sc) |  |
|  |  | MEA1 | MEA2 | MEA1 | MEA2 |
| MPPE data mode: Science | L mode | HDR(4s), Et-OMN(4s), INF(16s), Et-PAP(16s), VM(16s), 3D-LL(640s) | HDR(4s), Et-OMN(4s), INF(16s), Et-PAP(16s), VM(16s) |  |  |
|  | M mode (MPPEmode = 0-3 or 5) | HDR(4s), INF(4s), Et-OMNm(4s), Et-PAP(4s), VM(4s), 3D-M(8s) | HDR(4s), INF(4s) Et-OMNm(2s), Et-PAP(2s), VM(2s) |  |  |
|  | M mode (MPPE mode = 4 or 6-8) | HDR(4s), INF(4s), Et-OMNm(4s), Et-PAP(4s), VM(4s), 3D-M(4s) |  |  |  |
|  | H mode | HDR(4s),3D-H(4s) | HDR(4s),3D-H(2s) |  |  |
| Engineering | L mode | Same as L and Sci4&6-8 M mode |  | HDR(4s), Et-OMNe(4s), Et-PAPe(4s), INFe (4s) | HDR(4s), Et-OMNe(4s), Et-PAPe(4s), INFe (4s) |
|  | M mode |  |  | HDR(4s), Et-OMNe(2s), Et-PAPe(2s), INFe(4s) | HDR(4s), Et-OMNe(2s), Et-PAPe(2s), INFe(4s) |
|  | H0 mode | RAW-H0(4s) for M1-5 |  |  |  |

*L/M-data are continuously generated. The format is described in Appendix. All data are compressed by RICE code. Generating H-mode data is started by real-time CMD, TimeLine or trigger logic. Uses one-spin-average of potential data p and MGF data.

Table MDPSW-MEA-4 MEA data format

| Data products | Size (16 bits) | Total(B) |
| --- | --- | --- |
| M2 | [16sc] [32en] [16ch] | 16,384 |
| M4 | [16sc] [64en] [16ch] | 32,768 |

Table MDPSW-MEA-5 MEA EEPROM allocation for Look-up tables

| Table type | Size | Total(B) |
| --- | --- | --- |
| TMPC_lmt MCP_lmt IMCP_lmt IFPG_lmt IP5V_lmt | 1.5B + 1.5B (Lower and upper limit) | 15 |
| spare | 1B | 1 |
| 1/gf/eff | 1B[16ch] x 5(#1) | 80 |
| 1/gf/eff | 1B[16ch] x 5(#1) | 80 |
| Theta | 1B[16ch] x 5(#1) [+-11.25deg.] | 80 |
| Phi | 1B[16ch] x 5(#1) [+-11.25deg.] | 80 |
| Velocity32 T1-3 Factor | 2B[32en] x 3(#2) 1B[16ch] x 5(#1) | 192 80 |
| spare |  | 112 |
| Total |  | 720 |
| Total of MEA1&2 |  | 1440 |

#1: Number of the controlled MEA1&2 g-factor

#2: Energy tables are independent of g-factor mode.

1). ### _lmt:

When “HK_MON” is ENA and (HK_val < lmt[0] || HK_val > lmt[1])

MDP executes HV off (and PSU off).

2). 1/gf/eff

Used for calculating Et-OMN.

= MEA_GF0(0x200) + x (x >0) or 0 (x = 0)

3). 1/gf/eff

-Used for calculating VM

= MEA_GF0(0x200) + x (x >0) or 0 (x = 0)

4). Theta(radian)

Elevation[ch, gf] = -78.75 +s 22.5*ch + Theta[ch, gf] [+-11.25deg.].

5). Phi(radian)

Azimuth[ch, sc, gf] = Az0 + 22.5*sc + Phi[ch, gf] [+-11.25deg.].

6). Velocity

32 T1-3: = V[en]/V@30keV*0xffff;

7). Factor of V

= MEA_VF0(0x200) + x

Table MDPSW-MEA-6 MEA RAM (Look-up table)

| Table type | Size | Total(B) |
| --- | --- | --- |
| 1/gf/eff | 1B[16ch] x 5 for ET | 80 |
| 1/gf/eff | 1B[16ch] x 5 for VM | 80 |
| cos(theta) sin(theta) | 2B[16ch] x 5 2B[16ch] x 5 | 80 80 |
| cos(phi) sin(phi) | 2B[16sc][16ch] x 5 2B[16sc][16ch] x 5 | 1280 1280 |
| Velocity32 T1-3 Velocity64 T0 Factor | 2B[32en] x 3 -- 1B[16ch] x5 | 192 -- 80 |
| Total |  |  |
| Total of MEA1&2 |  |  |

Factor = 0.95 + +-0.05*1B

Table MDPSW-MEA-7 MEA product in the mission packet

| Data products | Format(16bits) | Size/Rate |
| --- | --- | --- |
| HDR(4s) | Header of the packet #1 | 8B/16bps |
| Et-OMN(4s) | [16en]  Et-OMNm(2s or 4s) (8bits) | 16B/32bps |
| Et-OMNe(4s) | [16en]  Et-OMNe(2s) (8bits) | 16B/32bps |
| INF(16s) | Same as INF(4s) | 8B/4bps |
| INFe(16s) | Same as INFe(4s) | 8B/4bps |
| Et-PAP(16s) | [4en][16ch] Et-PAP(2s or 4s) (8bits) | 64B/32bps |
| Et-PAPe(4s) | [16ch]  Et-PAPe(2s)[8 en][16 ch]x2 (8bits) | 16B/32bps |
| VM(16s) | [6en][n, nV(x, y, z), P(xx, yy, xx, xy, yz, xz),q(x, y, z)] [6en] 0: summing all en# above Pot*2; 1-5: 5 en# below Pot*2 | 156B/78bps |
| 3D-LL(640s) | [16 en] [88 dir] [16sc][8ch] | 2816B/35bps |
| Et-OMNm(2s) Et-OMNm(4s) | [32 en] x2 [8sc][16ch] + [8sc][16ch] (8bits) [32 en] [16sc][16ch] (8bits) | 64B/128bps 32B/64bps |
| Et-OMNe(2s) | [64 en] x2 [8sc][16ch] + [8sc][16ch] (8bits) | 128B/256bps |
| INF(4s) | 4B: width and en# of Et-PAP: 6b x 5 1B: sc# of Et-PAP 1B: Largest en# (2*potential) used for VM cal. 1B: en# of sweep stopping 1B: Energy table# | 8B/16bps |
| INFe(4s) | 7B: width and en# of Et-PAPe: 6b x 9 1B: sc# of Et-PAP | 8B/16bps |
| Et-PAP(2s) Et-PAP(4s) | [4en][16ch] x2 (8bits) [4en][16ch] (8bits) [4en]: 4 starting en# and width are selectable by CMD | 128B/256bps 64B/128bps |
| VM(2s) VM(4s) | [6 en][13vm] x2 [8sc][16ch] + [8sc][16ch] [6 en] [13vm] [16sc][8ch] [6en] 0: all en# above Pot*2; 1-5: 5 en# below Pot*2 | 312B/624bps 156B/312bps |
| Et-PAPe(2s) | [8 en] [16 ch] x2 [8sc][16ch] + [8sc][16ch] (8bits) | 256B/512bps |
| 3D-M(8s) | [16 en] [88 dir] [16sc][8ch] | 2816B/2816bps |
| 3D-M(4s) | [16 en] [88 dir] [16sc][8ch] | 2816B/5632bps |
| 3D-H(2s) 3D-H(4s) | [32 en] [88 dir] x2 [8sc][16ch] + [8sc][16ch] [32 en] [88 dir] [16sc][8ch] | 11265B/23kbps 5632B/11kbps |
| RAW-H0(4s) | Raw packet[8 + 1024B] x 16 or 32 |  |

#en: energy group; ch: channel group; sc: sector group; dir: direction group

88dir derived from 16ch x 8 sc at (2s) and from 8ch x 16sc at (4&8s).

Actual telemetry format is shown in Table MDPSW-MEA-8. Because of the 256-Byte Data Recorder (DR) segmentation, MDP sums up 16-spin L-mode data to 1 telemetry data. 8-byte comment is added as header to all the telemetry data, indicating ID, information regarding MPPE and MDP statuses and data sizes before and after compression. L, M and H-mode telemetry data are compressed by the rice code at “Prohibit product compression” = DIS(0).

In addition, 8-bit counter Et-OMN and Et-PAP are derived from 32-bit and 16-bit counters by using 32-bit linear to 8-bit log and 16-bit linear to 8-bit log algorithms, respectively (Table MDPSW-MEA-9, 10). VM which is originally 64bit double array is converted to 16bit float array (Table MDPSW-MEA-11). Direction group in counter[88dir] is defined as shown in Table MDPSW-MEA-12, converted from counter[8ch][16sc] of channel and sector groups.

Table MDPSW-MEA-8 MEA Telemetry format

●：MDP info indicates whether the data is included or not.

△：                     the data is Snapshot/average.

3D-LL and 3D-M is produced by MEA1.

Comment

| Item | Byte | Note |
| --- | --- | --- |
| Header ID | 2 | 0xca 0xe1(MEA1), 0xca 0xe2(MEA2) |
| MPPE mode | 1 | [0]: 0/1 = average/snapshot [1]: 0/1 = SCI/ENG [2-5]: Mode (0-8) [6-7]: 0/1 RICE_UINT16/8 |
| MDP Info. | 1 | Described in the following |
| Original size | 2 | Product size |
| Compressed size | 2 |  |
| Total | 8 |  |

# L mode

SCI-M2-0 (MEA1&2) (16-spin products  1360B )

| Product | Byte | Note |
| --- | --- | --- |
| MPPE mode | 1 | [0]: 0/1 = average/snapshot [1]: 0/1 = SCI/ENG [2-5]: Mode (0-8) [6-7]: spare |
| MDPInfo. | 1 | [0-3]: Data ID = 0 [4]: Potential mode [5]: include HDR(16s), VM(16s) & Et(16s) [4-7]: spare |
| Size | 2 | =26/254 |
| HDR(4s) | 8 |  |
| Et-OMN(4s) | 16 |  |
| INF(16s) ● | 8 |  |
| Et-PAP(16s) ●△ | 64 | /4spin |
| VM(16s) ●△ | 156 | /4spin |
| Total |  | 28/256 |

SCI-M2-a (MEA1)

| Product | Byte | Note |
| --- | --- | --- |
| MDPInfo. | -- | [0-3]: Data ID = 3 [4-7]: spare |
| HDR(4s) | 8 |  |
| 3D-LL(640s) ● | 2816 |  |
| Total | 2824 |  |

ENG-M4 (MEA1&2) (16-spin products  832B)

| Product | Byte | Note |
| --- | --- | --- |
| MPPE mode | 1 | [0]: 0/1 = average/snapshot [1]: 0/1 = SCI/ENG [2-5]: Mode (0-8) [6-7]: spare |
| MDPInfo. | 1 | [0-3]: Data ID = 2 [4]: Potential mode [5-7]: |
| Size | 2 | =52 |
| HDR(4s) | 8 |  |
| INFe(4s) | 8 |  |
| Et-OMNe(4s) | 16 |  |
| Et-PAPe(4s) | 16 |  |
| Total | 52 |  |

SCI-M*(MEA1&2) (16-spin products  704B)  *Only the 1st packet is sent except M2&M4

| Product | Byte | Note |
| --- | --- | --- |
| MPPE mode | 1 | [0]: 0/1 = average/snapshot [1]: 0/1 = SCI/ENG [2-5]: Mode (0-8) [6-7]: spare |
| MDPInfo. | 1 | [0-3]: Data ID = 7 [4]: Potential mode [5-7]: |
| Size | 2 | =44 |
| HDR(4s) | 8 |  |
| Packet(4s) | 32 | CH0 and SW0 |
| Total | 44 |  |

# M mode

SCI -M2-0 (MEA1)

| Product | Byte | Note |
| --- | --- | --- |
| MDP info. | -- | [0-3]: Data ID = 0 [4]: Potential mode [5-7]: spare |
| HDR | 8 |  |
| INF (4s) | 8 |  |
| Et-OMN(4s) | 32 |  |
| Et-PAP(4s) | 64 |  |
| VM(4s) | 156 |  |
| Total | 268 |  |

SCI -M2-a (MEA1)

| Product | Byte | Note |
| --- | --- | --- |
| MDP info. | -- | [0-3]: Data ID = 3 [4-7]: spare |
| HDR | 8 |  |
| 3D-M(8s) △ or 3D-M(4s) | 2816 | MPPE mode = 1-3, 5 MPPE mode = 4, 6-8 |
| Total | 2824 |  |

SCI-M2-1 (MEA2)

| Product | Byte | Note |
| --- | --- | --- |
| MDP info. |  | [0-3]: Data ID = 1 [4]: Potential mode [5-7]: spare |
| HDR | 8 | Header of the packet #1 |
| INF (4s) | 8 |  |
| Et-OMN(2s) | 32 x2 |  |
| Et-PAP(2s) | 64 x2 |  |
| VM(2s) | 156 x2 |  |
| Total | 520 |  |

ENG-M4

| Product | Byte | Note |
| --- | --- | --- |
| MDP info. |  | [0-3]: Data ID = 2 [4]: Potential mode [5-7]: spare |
| HDR | 8 | Header of the packet #1 |
| INFe(4s) | 8 |  |
| Et-OMNe(2s) | 64 x2 |  |
| Et-PAPe(2s) | 128 x2 |  |
| Total | 400 |  |

SCI-M*(MEA1&2) *Except M2 and M4, the first packet of MEA is included.

| Product | Byte | Note |
| --- | --- | --- |
| MDPInfo. |  | [0-3]: Data ID = 7 [4]: Potential mode [5-7]: |
| HDR(4s) | 8 |  |
| Packet(4s) | 1024 | CH0-15 at SW0 |
| Total | 1032 |  |

# H mode (MEA1)

| Product | Byte | Note |
| --- | --- | --- |
| MDP info. |  | Data ID = 0 |
| HDR | 8 | Header of the packet #1 |
| 3D-H(4s) | 5632 |  |
| Total | 5640 |  |

# H mode (MEA2)

| Product | Byte | Note |
| --- | --- | --- |
| MDP info. |  | Data ID = 0, 1 |
| HDR | 8 |  |
| 3D-H(2s) | 5632 | 0 : CH0-7 1: CH8-15 |
| Total | 5640 x2 |  |

# H0 mode

| Product | Byte | Note |
| --- | --- | --- |
| MDP info. |  | Data ID = 4 |
| RAW-H0 | 8 + 1024 | Raw packet |
| Total | 1032 | 16 or 32 packets during 1spin |

Table MDPSW-MEA-9 32bit linear to 8 bit log from the ENA application

| Linear input code (32 bit) | Compressed code (8 bit) |
| --- | --- |
| 00000000 00000000 00000000 000tuvwx | 00 0tuvwx |
| 00000000 00000000 00000000 001uvwxy | 00 10uvwx |
| 00000000 00000000 00000000 01uvwxya | 00 110uvw |
| 00000000 00000000 00000000 1uvwxyab | 00 111uvw |
| 00000000 00000000 00000001 uvwabcde | 01 000uvw |
| 00000000 00000000 0000001u vwabcdef | 01 001uvw |
| 00000000 00000000 000001uv wabcdefg | 01 010uvw |
| 00000000 00000000 00001uvw abcdefgh | 01 011uvw |
| 00000000 00000000 0001uvwa bcdefghj | 01 100uvw |
| 00000000 00000000 001uvwab cdefghij | 01 101uvw |
| 00000000 00000000 01uvwabc defghijk | 01 110uvw |
| 00000000 00000000 1uvwabcd efghijkl | 01 111uvw |
| 00000000 00000001 uvwabcde fghijklm | 10 000uvw |
| 00000000 0000001u vwabcdef ghijklmn | 10 001uvw |
| 00000000 000001uv wabcdefg hijklmno | 10 010uvw |
| 00000000 00001uvw abcdefgh ijklmnop | 10 011uvw |
| 00000000 0001uvwa bcdefghi jklmnopq | 10 100uvw |
| 00000000 001uvwab cdefghij klmnopqr | 10 101uvw |
| 00000000 01uvwabc defghijk lmnopqrs | 10 110uvw |
| 00000000 1uvwabcd efghijkl mnopqrst | 10 111uvw |
| 00000001 uvwabcde fghijklm nnnnnnnn | 11 000uvw |
| 0000001u vwabcdef ghijklmn nnnnnnnn | 11 001uvw |
| 000001uv wabcdefg hijklmno nnnnnnnn | 11 010uvw |
| 00001uvw abcdefgh ijklmnop nnnnnnnn | 11 011uvw |
| 0001uvwa bcdefghi jklmnopq nnnnnnnn | 11 100uvw |
| 001uvwab cdefghij klmnopqr nnnnnnnn | 11 101uvw |
| 01uvwabc defghijk lmnopqrs nnnnnnnn | 11 110uvw |
| 1uvwabcd efghijkl mnopqrst nnnnnnnn | 11 111uvw |

Table MDPSW-MEA-10 16bit linear to 8 bit log from the ENA application

| Linear input code (16 bit) | Compressed code (8 bit) |
| --- | --- |
| 00000000 000uvwxy | 000uvwxy |
| 00000000 001uvwxy | 001uvwxy |
| 00000000 01uvwxya | 010uvwxy |
| 00000000 1uvwxyab | 011uvwxy |
| 00000001 uvwxabcd | 1000uvwx |
| 0000001u vwxabcde | 1001uvwx |
| 000001uv wxabcdef | 1010uvwx |
| 00001uvw xabcdefg | 1011uvwx |
| 0001uvwx abcdefgh | 1100uvwx |
| 001uvwxa bcdefghi | 1101uvwx |
| 01uvwxab cdefghij | 1110uvwx |
| 1uvwxabc defghijk | 1111uvwx |

Table MDPSW-MEA-11 16 bit float (IEEE 754 standard)

| sign | Expo. 5bit |  |  |  |  | Fraction 10bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 |  |  |  |  | 10 | 9 |  |  |  |  |  |  |  |  | 0 |

Table MDPSW-MEA-12 8ch x 16sc to 88dir

|  |  | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  |
| 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 |
| 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 |
| 76 |  | 77 |  | 78 |  | 79 |  | 80 |  | 81 |  | 82 |  | 83 |  |
|  |  | 84 |  |  |  | 85 |  |  |  | 86 |  |  |  | 87 |  |

5．Common FM header (10B)

MDP common function insert common header of 10 Bytes following the CCSDS primary and secondary headers. For MEA, “TI-Data” includes the TI derived from the header of the last (16th or 32nd) packet (MDP SW was revised based on the test 15-Dec 2017), while HDR(4s) in the mission data contains the header of packet#1.

NodeID/Data-ID	Mode-ID	*TI-Data		Counter	Size-Comment	Size-Data

1B		1B	4B (MSB>LSB)	1B 	1B		2B (MSB>LSB)