MPPE/MEA MDP Software Spec

Bepi-Colombo MEA & MDP Team

Ver.6.1

2014.03.20

*Shared data

short Gs_TRGBUF_EFD_Pot[ ]; from EFD (1-spin average)

short Gs_TRGBUF_MGFO_Bx[ ] from MGFO – Bx (1-spin average)

short Gs_TRGBUF_MGFO_By[ ] from MGFO – By (1-spin average)

short Gs_TRGBUF_MGFO_Bz[ ] from MGFO – Bz (1-spin average)

unsigned short Gus_TRGBUF_MEA_T[ ]; from MEA

０．Open

１．Development system

MDP:		Kasaba（Tohoku Univ.）

MPPE:		Yokota（ISAS）

２．Command: MEA task in MDP SW

CMD：	Ground　>　DMC　>　MDP(MEA1&2)

| CMD (MD1U_**) MEA1_: 0x05; MEA2_0x06 |  | HK# | Description |
| --- | --- | --- | --- |
| MEA1/2_INIT | 0x0030 |  | Initialize SW parameters |
| MEA1/2_ERR_CLR | 0x0031 |  | Clear the counters |
| MEA1/2_ROM_READ | 0x0032 | 17 | Obtain the parameters from E2PROM |
| MEA1/2_PROC_SNAPSHOT | 0x0010 | 3 | Select SNAPSHOT for multi-spin products |
| MEA1/2_PROC_ AVERAGE | 0x0011 | 3 | Select AVERAGE for multi-spin products |
| MEA1/2_DATA_MODE zz xx | 0x0012 | 3 | Set MPPE mode zz = 0/1  SCI/ENG; xx = 0-8 for SCI and 0-1 for ENG |
| MEA1/2_POT_MODE 00 xx | 0x0013 | 4 | Set the mode for the satellite potential xx = 00/01  0/Sp1 (Sp1 = A*p + B ) |
| MEA1/2_POT_PARAM zz xx aa bb | 0x0015 | 5 | Set A of Sp1 for the satellite potential A= (z << 8) +xx; B = (a << 8) +bb (12bits) #1 |
| MEA1/2_SET_ETPAP zz xx aa bb cc dd ee ff gg | 0x0016 | 12 | Set en# of EtPAP[4en][16ch] and EtPAPe[8en][16ch] 4 or 8 start points of en# and width: zz: width (zz = 0  1 en) en = xx aa bb cc (& 0x1f) used for EtPAP; en = xx aa bb cc dd ee ff gg (& 0x3f) used for EtPAPe; en# = xx - xx + zz, aa - aa + zz, … |
| MEA1/2_SET_VM_AL zz xx aa bb | 0x0018 | 14 | Set the flag of using energy step # for VM[0](…) zz: 31-24; xx: 23-16; aa: 15-8; bb: 7-0# of 32 energy steps 0/1  NOT USE/USE Ex. “ff ff ff fe” does not use the energy step# of 0. |
| MEA1/2_SET_ETBL 00 xx | 0x0019 | 6 | Set the table# of energy(velocity) sweep for VM cal. xx = 1-3 T1-T3 (if xx = 0, xx goes to 1.) Should follow the MEA CMD of changing energy sweep table. |
| MEA1/2_SET_HK_MON 00 xx | 0x0020 | 6 | Set MEA1/2 HK monitoring 1/0 ENA/DIS xx = [][][][4][3][2][1][0] (ex. 00 00  all DIS, 00 1f  all ENA) [0]: MEA1/2_HK_TMP_MCP [1]: MEA1/2_HK_MCP [2]: MEA1/2_I_MCP [3]: MEA1/2_HK_TMP_FPG [4]: MEA1/2_HK_I_P5V |
| MEA1/2_SET_TMCP_LIM zz xx aa bb | 0x0021 | 7 | Set the lower/upper limits of MEA1/2_HK_TMP_MCP (z << 8) +xx : lower limit; (a << 8) + bb : upper limit (12bits) |
| MEA1/2_SET_MCP_LIM zz xx aa bb | 0x0022 | 8 | Set the lower/upper limits of MEA1/2_HK_MCP (z << 8) +xx : lower limit; (a << 8) + bb : upper limit (12bits) |
| MEA1/2_SET_IMCP_LIM zz xx aa bb | 0x0023 | 9 | Set the lower/upper limits of MEA1/2_I_MCP (z << 8) +xx : lower limit; (a << 8) + bb : upper limit (12bits) |
| MEA1/2_SET_TFPG_LIM zz xx aa bb | 0x0024 | 10 | Set the lower/upper limits of MEA1/2_HK_TMP_FPG (z << 8) +xx : lower limit; (a << 8) + bb : upper limit (12bits) |
| MEA1/2_SET_IP5V_LIM zz xx aa bb | 0x0025 | 11 | Set the lower/upper limits of MEA1/2_HK_I_P5V (z << 8) +xx : lower limit; (a << 8) + bb : upper limit (12bits) |
| MEA1/2_SET_STPE_AUT 00 xx | 0x0026 | 3 | Set automatic sending of ‘MEA1/2_StpEnMd_ON’ENA/DIS xx = 1/0 ENA/DIS In case, MDP sends MEA1/2_ StpEnMd_ON (0x11) 0x01-0x20 |
| MEA1/2_SND_GF_AUT zz xx | 0x0027 | 3 15 | Set automatic sending of ‘MEA1/2_G_Factor’ ENA/DIS zz: X (number of spin); xx = 1/0 ENA/DIS In case, MDP sends MEA1/2_G_Factor(0x08) 0x00-0x04 |
| MEA1/2_SET_MCPV zz xx | 0x0028 | 17 | Set V1 and V2 to use before MDP turn off the MCP zz: V1; xx: V2 |
| MEA1/2_ETP_MODE zz xx | 0x0029 | 18 | Set the mode for sector# of EtPAP zz = 1/0  use sector # from B(MGF) + offset(xx)/xx(0-7) |
| MEA1/2_CNT1_SET zz xx aa bb … (for debug) | 0x0002 | 18 | 00 01 aa bb cc dd ee: #SW cal aa(0/1) = DIS/ENA; bb = en; cc = ch; dd = sc; ee = count 00 04 aa bb: #prohibit product compression if (bb) 00 02 aa bb: #send CMD(aa bb) to MEA 00 07 00 bb #limit the size of H-mode product if (bb): |

MEA1/2 changes the parameters of MEA1 or MEA2.

#1) Actual value(V) = (VAL – 0x07ff) x 100/0x07ff (VAL = A, or B)

CMD：	MDP　>　MEA

| CMD | Description |
| --- | --- |
| MEA1/2_StpEnMd_ON 0x11 aa(energy) aa = 0-63 | #Send the energy step number at each spin The energy number is determined by the Satellite Pot x 2 – 5 steps (to protect the MCP from photoelectrons) |
| MEA1/2_G_Factor 0x08 aa(GF_value) aa = 0x00-0x04 | #Decrease g-factor when: Count > 10^5/s/ch continuously during X spins # Increase g-factor when: Count/spin/16chs < 1000 continuously during X spins (X is changeable by CMD.) MEA1: [default  20  60  250  1000] MEA2: [default  1  3  20  60] *GF1&T3 *When MEA1 backs to 60, MEA2 also goes to 60. |

Emergency

３．HK

・HK		20 Byte

-MDP monitors the following parameters at 1Hz.

-MDP sends CMDs when HK_mon = ENA && (HK < lower limit || HK > upper limit).

MEA1/2_HK_TMP_MCP (Upper/lower limits are changeable by CMD)

MEA1/2_HK_MCP (Upper/lower limits are changeable by CMD)

MEA1/2_I_MCP (Upper/lower limits are changeable by CMD)

#MEA1/2_MCP_ON(0x1c) V1

#wait for 4s

#MEA1/2_MCP_ON(0x1c) V2

#wait for 4s

#MEA1/2_MCP_ON(0x1c) 0

#wait for 4s

#MEA1/2_MCP_OFF (0x1900)

MEA1/2_HK_TMP_FPG (Upper/lower limits are changeable by CMD)

MEA1/2_HK_I_P5V (Upper/lower limits are changeable by CMD)

# MEA1/2_MCP_ON(0x1c) V1  V2  0  MEA1/2_MCP_OFF(0x1900) as the above

#wait for 12 s

#MEA1/2 PSU OFF (DmcRequestCmd)

・User-HK	128 Bytes	(No parameter is monitored by MDP)

・Software HK	52Bytes        MEA1/2: Individual parameters

| Name | Description | Byte |
| --- | --- | --- |
| 0. MEA1/2_S_CMD_CTR | Software Command counter | 1 |
| 1. MEA1/2_S_ERR_CTR | Software error counter | 1 |
| 2. MEA1/2_S_CMD_ANS | Software Command answer | 4 |
| 3. MEA1/2_S_DATA_MODE | [7]: 0/1 = DIS/ENA SND_GF_AUT [6]: 0/1 = DIS/ENA SND_STPE_AUT [2-5]: Mode [1]: 0/1 = SCI/EN [0]: 0/1 = average/snapshot | 1 |
| 5. MEA1&2_S_POT_PARAM | 1.5B: A; 1.5B: B | 3 |
| 6. MEA1/2_S_HK_MON (& Energy table number) | [7] POT_MODE [5-6]: Energy table number [4]: MEA1/2_HK_I_P5V [3]: MEA1/2_HK_TMP_FPG [2]: MEA1/2_I_MCP [1]: MEA1/2_HK_MCP [0]: MEA1/2_HK_TMP_MCP 0/1 = DIS/ENA | 1 |
| 7. MEA1/2_S_TMCP_LIM | Lower limit and upper limit (1.5B+1.5B) | 3 |
| 8. MEA1/2_S_MCP_LIM | Lower limit and upper limit | 3 |
| 9. MEA1/2_S_IMCP_LIM | Lower limit and upper limit | 3 |
| 10. MEA1/2_S_TFPG_LIM | Lower limit and upper limit | 3 |
| 11. MEA1/2_S_IP5V_LIM | Lower limit and upper limit | 3 |
| 12. MEA1/2_S_ETPAP | zz xx aa bb cc dd ee ff gg zz: width; others: starting points ex. for (i = xx; i < xx + zz; ++i) | 8->9 |
| 13. MEA1/2_S_VM | zz xx aa bb cc dd | 6->0 |
| 14. MEA1/2_S_VM_AL | zz xx aa bb | 4 |
| 15. MEA1/2_S_GF_AVAL | X (spin number) for g-factor control | 1 |
| 16. MEA_S_HK_ANMLY (MDP did [5] or detected [4-1] is out of the range) | [5]: PSU-OFF [4]: MEA1/2_HK_I_P5V [3]: MEA1/2_HK_TMP_FPG [2]: MEA1/2_I_MCP [1]: MEA1/2_HK_MCP [0]: MEA1/2_HK_TMP_MCP | 1 |
| 17 MEA1/2_S_MCPV | V1 and V2 to set before MDP turn off the MCP | 2 |
| 18. MEA1/2_S_PARAM | 0-[7] spare 0-[6]: B(MGF) Mode (1/0: Use MGF_B/CMD_B) 0-[5-3]: B(sector) for EtPAP 0-[2]: prohibit product compression 0-[1]: SWcal ENA(1)/DIS(0) 0-[0]: Read the parameters of E2PROM (TRUE/FALSE) 0/1 = DIS/ENA 1: 0xca 2: 0xe1(MEA1), 0xe2(MEA2) | 3 |
|  | TOTAL | 46 |

MEA1/2: Individual paremeters

４．TLM

MDP products（MEA1 & 2）

|  |  | MEA operation mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | M2 (32en x 16sc) |  | M4 (64en x 16sc) |  |
|  |  | MEA1 | MEA2 | MEA1 | MEA2 |
| MPPE data mode: Science | L mode | HDR(4s), Et-OMN(4s), INF(16s), Et-PAP(16s), VM(16s), 3D-LL(640s) | HDR(4s), Et-OMN(4s), INF(16s), Et-PAP(16s), VM(16s) |  |  |
|  | M mode (MPPEmode = 0-3 or 5) | HDR(4s), INF(4s), Et-OMNm(4s), Et-PAP(4s), VM(4s), 3D-M(8s) | HDR(4s), INF(4s) Et-OMNm(2s), Et-PAP(2s), VM(2s) |  |  |
|  | M mode (MPPE mode = 4 or 6-8) | HDR(4s), INF(4s), Et-OMNm(4s), Et-PAP(4s), VM(4s), 3D-M(4s) |  |  |  |
|  | H mode | HDR(4s),3D-H(4s) | HDR(4s),3D-H(2s) |  |  |
| Engineering | L mode | Same as L and Sci4&6-8 M mode |  | HDR(4s), Et-OMNe(4s), Et-PAPe(4s), INFe (4s) | HDR(4s), Et-OMNe(4s), Et-PAPe(4s), INFe (4s) |
|  | M mode |  |  | HDR(4s), Et-OMNe(2s), Et-PAPe(2s), INFe(4s) | HDR(4s), Et-OMNe(2s), Et-PAPe(2s), INFe(4s) |
|  | H0 mode | RAW-H0(4s) for M1-5 |  |  |  |

Note:

*L/M-data are continuously generated. The format is described in Appendix. All data are compressed by RICE code.

*Generating H-mode data is started by real-time CMD, TimeLine or trigger logic.

*Uses one-spin-average of potential data p and MGF data.

＊E2PROM for Look-up tables

| Table type | Size | Total(B) |
| --- | --- | --- |
| TMPC_lmt MCP_lmt IMCP_lmt IFPG_lmt IP5V_lmt | 1.5B + 1.5B (Lower and upper limit) | 15 |
| spare | 1B | 1 |
| 1/gf/ | 1B[16ch] x 5(#1) | 80 |
| 1/gf/ | 1B[16ch] x 5(#1) | 80 |
| Theta | 1B[16ch] x 5(#1) [+-11.25deg.] | 80 |
| Phi | 1B[16ch] x 5(#1) [+-11.25deg.] | 80 |
| Velocity32 T1-3 Factor | 2B[32en] x 3(#2) 1B[16ch] x 5(#1) | 192 80 |
| spare |  | 112 |
| Total |  | 720 |
| Total of MEA1&2 |  | 1440 |

#1: Number of the controlled MEA1&2 g-factor

#2: Energy tables are independent of g-factor mode.

1). ### _lmt:

When “HK_MON” is ENA and (HK_val < lmt[0] || HK_val > lmt[1])

MDP executes HV off (and PSU off).

2). 1/gf/

Used for calculating Et-OMN.

= MEA_GF0(0x200) + x (x >0) or 0 (x = 0)

3). 1/gf/

-Used for calculating VM

= MEA_GF0(0x200) + x (x >0) or 0 (x = 0)

4). Theta(radian)

Elevation[ch, gf] = -78.75 +s 22.5*ch + Theta[ch, gf] [+-11.25deg.].

5). Phi(radian)

Azimuth[ch, sc, gf] = Az0 + 22.5*sc + Phi[ch, gf] [+-11.25deg.].

6). Velocity

32 T1-3: = V[en]/V@30keV*0xffff;

7). Factor of V

= MEA_VF0(0x200) + x

＊RAM(Look-up table

| Table type | Size | Total(B) |
| --- | --- | --- |
| 1/gf/ | 1B[16ch] x 5 for ET | 80 |
| 1/gf/ | 1B[16ch] x 5 for VM | 80 |
| cos(theta) sin(theta) | 2B[16ch] x 5 2B[16ch] x 5 | 80 80 |
| cos(phi) sin(phi) | 2B[16sc][16ch] x 5 2B[16sc][16ch] x 5 | 1280 1280 |
| Velocity32 T1-3 Velocity64 T0 Factor | 2B[32en] x 3 -- 1B[16ch] | 192 -- 16 |
| Total |  |  |
| Total of MEA1&2 |  |  |

Factor = 0.95 + +-0.05*1B

＊MEA1&2 data format

| Data products | Size (16 bits) | Total(B) |
| --- | --- | --- |
| M2 | [16sc] [32en] [16ch] | 16,384 |
| M4 | [16sc] [64en] [16ch] | 32,768 |

A．Data Format

＊MDP products

| Data products | Format(16bits) | Size/Rate |
| --- | --- | --- |
| HDR(4s) | Header of the packet #1 | 8B/16bps |
| Et-OMN(4s) | [16en]  Et-OMNm(2s or 4s) (8bits) | 16B/32bps |
| Et-OMNe(4s) | [16en]  Et-PAPe(2s) (8bits) | 16B/32bps |
| INF(16s) | Same as INF(4s) | 8B/4bps |
| INFe(16s) | Same as INFe(4s) | 8B/4bps |
| Et-PAP(16s) | [4en][16ch] Et-PAP(2s or 4s) (8bits) | 64B/32bps |
| Et-PAPe(4s) | [16ch]  Et-PAPe(2s)[8 en][16 ch]x2 (8bits) | 16B/32bps |
| VM(16s) | [6en][n, nV(x, y, z), P(xx, yy, xx, xy, yz, xz),q(x, y, z)] [6en] 0: summing all en# above Pot*2; 1-5: 5 en# below Pot*2 | 156B/78bps |
| 3D-LL(640s) | [16 en] [88 dir] [16sc][8ch] | 2816B/35bps |
| Et-OMNm(2s) Et-OMNm(4s) | [32 en] x2 [8sc][16ch] + [8sc][16ch] (8bits) [32 en] [16sc][16ch] (8bits) | 64B/128bps 32B/64bps |
| Et-OMNe(2s) | [64 en] x2 [8sc][16ch] + [8sc][16ch] (8bits) | 128B/256bps |
| INF(4s) | 4B: width and en# of Et-PAP: 6b x 5 1B: sc# of Et-PAP 1B: Largest en# (2*potential) used for VM cal. 1B: en# of sweep stopping 1B: Energy table# | 8B/16bps |
| INFe(4s) | 7B: width and en# of Et-PAPe: 6b x 9 1B: sc# of Et-PAP | 8B/16bps |
| Et-PAP(2s) Et-PAP(4s) | [4en][16ch] x2 (8bits) [4en][16ch] (8bits) [4en]: 4 starting en# and width are selectable by CMD | 128B/256bps 64B/128bps |
| VM(2s) VM(4s) | [6 en][13vm] x2 [8sc][16ch] + [8sc][16ch] [6 en] [13vm] [16sc][8ch] [6en] 0: all en# above Pot*2; 1-5: 5 en# below Pot*2 | 312B/624bps 156B/312bps |
| Et-PAPe(2s) | [8 en] [16 ch] x2 [8sc][16ch] + [8sc][16ch] (8bits) | 256B/512bps |
| 3D-M(8s) | [16 en] [88 dir] [16sc][8ch] | 2816B/2816bps |
| 3D-M(4s) | [16 en] [88 dir] [16sc][8ch] | 2816B/5632bps |
| 3D-H(2s) 3D-H(4s) | [32 en] [88 dir] x2 [8sc][16ch] + [8sc][16ch] [32 en] [88 dir] [16sc][8ch] | 11265B/23kbps 5632B/11kbps |
| RAW-H0(4s) | Raw packet[8 + 1024B] x 16 or 32 |  |

#en: energy group; ch: channel group; sc: sector group; dir: direction group

88dir derived from 16ch x 8 sc at (2s) and from 8ch x 16sc at (4&8s).

＊TLM format

●：MDP info indicates whether the data is included or not.

△：                     the data is Snapshot/average.

3D-LL and 3D-M is produced by MEA1.

Comment

| Item | Byte |  |
| --- | --- | --- |
| Header ID | 2 | 0xca 0xe1(MEA1), 0xca 0xe2(MEA2) |
| MPPE mode | 1 | [0]: 0/1 = average/snapshot [1]: 0/1 = SCI/ENG [2-5]: Mode (0-8) [6-7]: 0/1 RICE_UINT16/8 |
| MDP Info. | 1 | Described in the following |
| Original size | 2 | Product size |
| Compressed size | 2 |  |
| Total | 8 |  |

# L mode

SCI-M2-0 (MEA1&2) (16-spin products  1360B )

| Product | Byte |  |
| --- | --- | --- |
| MPPE mode | 1 | [0]: 0/1 = average/snapshot [1]: 0/1 = SCI/ENG [2-5]: Mode (0-8) [6-7]: spare |
| MDPInfo. | 1 | [0-3]: Data ID = 0 [4]: Potential mode [5]: include HDR(16s), VM(16s) & Et(16s) [4-7]: spare |
| Size | 2 | =26/254 |
| HDR(4s) | 8 |  |
| Et-OMN(4s) | 16 |  |
| INF(16s) ● | 8 |  |
| Et-PAP(16s) ●△ | 64 | /4spin |
| VM(16s) ●△ | 156 | /4spin |
| Total |  | 28/256 |

SCI-M2-a (MEA1)

| Product | Byte |  |
| --- | --- | --- |
| MDPInfo. | -- | [0-3]: Data ID = 3 [4-7]: spare |
| HDR(4s) | 8 |  |
| 3D-LL(640s) ● | 2816 |  |
| Total | 2824 |  |

ENG-M4 (MEA1&2) (16-spin products  832B)

| 項目 | Byte | 内容 |
| --- | --- | --- |
| MPPE mode | 1 | [0]: 0/1 = average/snapshot [1]: 0/1 = SCI/ENG [2-5]: Mode (0-8) [6-7]: spare |
| MDPInfo. | 1 | [0-3]: Data ID = 2 [4]: Potential mode [5-7]: |
| Size | 2 | =52 |
| HDR(4s) | 8 |  |
| INFe(4s) | 8 |  |
| Et-OMNe(4s) | 16 |  |
| Et-PAPe(4s) | 16 |  |
| Total | 52 |  |

SCI-M*(MEA1&2) (16-spin products  B)  *M2, M4 以外は最初のパケット出力

| 項目 | Byte | 内容 |
| --- | --- | --- |
| MPPE mode | 1 | [0]: 0/1 = average/snapshot [1]: 0/1 = SCI/ENG [2-5]: Mode (0-8) [6-7]: spare |
| MDPInfo. | 1 | [0-3]: Data ID = 7 [4]: Potential mode [5-7]: |
| Size | 2 | =44 |
| HDR(4s) | 8 |  |
| Packet(4s) | 32 | CH0 and SW0 |
| Total | 44 |  |

# M mode

SCI -M2-0 (MEA1)

|  | Byte | Note |
| --- | --- | --- |
| MDP info. | -- | [0-3]: Data ID = 0 [4]: Potential mode [5-7]: spare |
| HDR | 8 |  |
| INF (4s) | 8 |  |
| Et-OMN(4s) | 32 |  |
| Et-PAP(4s) | 64 |  |
| VM(4s) | 156 |  |
| Total | 268 |  |

SCI -M2-a (MEA1)

|  | Byte | Note |
| --- | --- | --- |
| MDP info. | -- | [0-3]: Data ID = 3 [4-7]: spare |
| HDR | 8 |  |
| 3D-M(8s) △ or 3D-M(4s) | 2816 | MPPE mode = 1-3, 5 MPPE mode = 4, 6-8 |
| Total | 2824 |  |

SCI-M2-1 (MEA2)

|  | Byte | Note |
| --- | --- | --- |
| MDP info. |  | [0-3]: Data ID = 1 [4]: Potential mode [5-7]: spare |
| HDR | 8 | Header of the packet #1 |
| INF (4s) | 8 |  |
| Et-OMN(2s) | 32 x2 |  |
| Et-PAP(2s) | 64 x2 |  |
| VM(2s) | 156 x2 |  |
| Total | 520 |  |

ENG-M4

|  | Byte | Note |
| --- | --- | --- |
| MDP info. |  | [0-3]: Data ID = 2 [4]: Potential mode [5-7]: spare |
| HDR | 8 | Header of the packet #1 |
| INFe(4s) | 8 |  |
| Et-OMNe(2s) | 64 x2 |  |
| Et-PAPe(2s) | 128 x2 |  |
| Total | 400 |  |

SCI-M*(MEA1&2) (16-spin products  B)  *M2, M4 以外は最初のパケット出力

| 項目 | Byte | 内容 |
| --- | --- | --- |
| MDPInfo. |  | [0-3]: Data ID = 7 [4]: Potential mode [5-7]: |
| HDR(4s) | 8 |  |
| Packet(4s) | 1024 | CH0-15 at SW0 |
| Total | 1032 |  |

# H mode (MEA1)

|  | Byte | Note |
| --- | --- | --- |
| MDP info. |  | Data ID = 0 |
| HDR | 8 | Header of the packet #1 |
| 3D-H(4s) | 5632 |  |
| Total | 5640 |  |

# H mode (MEA2)

|  | Byte | Note |
| --- | --- | --- |
| MDP info. |  | Data ID = 0, 1 |
| HDR | 8 |  |
| 3D-H(2s) | 5632 | 0 : CH0-7 1: CH8-15 |
| Total | 5640 x2 |  |

# H0 mode

|  | Byte | Note |
| --- | --- | --- |
| MDP info. |  | Data ID = 4 |
| RAW-H0 | 8 + 1024 | Raw packet |
| Total | 1032 | 16 or 32 packets during 1spin |

＊88 dir from 8ch x 16sc

|  |  | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  | 11 |  |
| 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 |
| 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 |
| 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 |
| 76 |  | 77 |  | 78 |  | 79 |  | 80 |  | 81 |  | 82 |  | 83 |  |
|  |  | 84 |  |  |  | 85 |  |  |  | 86 |  |  |  | 87 |  |